| Generic Risk Assessment: |  |  |  | Management of Transport Fleet and Drivers |  |  |  |  |  |  |  | Assessment No: |  | CVTRA: 001 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description of Activity: |  |  | Vehicle Trust that own vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |
| People at Risk: |  | Passengers | Yes |  | Volunteers: | Yes | Drivers: | Yes | Carers: | Maybe | Lone Workers: |  | Yes |  | Public: | Yes |
| Injuries: | Anxiety, back/head injury, bruise/cut, burn, choke, fatality, fracture, puncture wound, sprain/strain, suffocate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Hazard |  | Accident | Likelihood | Severity | Risk Factor | Control Measures | Likelihood |  | Severity |  | Risk Factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - Insurance - WoF, CoF - Servicing - Periodical garage safety inspection - Equipment | Collision, breakdown | 1 3 3 3 3 | 3 3 3 3 3 | 3 9 9 9 9 | - Vehicle and drivers insured for all purposes for which the vehicle is used - Annual WoF, CoF test: - Vehicle serviced/maintained in accordance with manufacturers schedule and insurance requirement - Periodic 12-week inspection unnecessary if: - mileage and use of vehicle is low - up to date log book is in place (see CVT: 013 for details) - monthly safety checklist is completed as laid out in section 2b (see CVT: 014 for details) - defect reporting system is used (see CVT: 015 for details) Reviewed if: usage of vehicle changes or above procedures not followed. - Vehicle Trust must review whether equipment outside vehicle standard is needed i.e. rear-view monitoring system, extra mirrors, dashcam, etc… | 1 1 1 1 1 |  | 3 3 3 1 3 |  | 3 3 3 1 3 |
| Hazard |  | Accident | L | S | RF | Control Measures |  | L |  | S | RF |
| 2 | - Log book system - Safety check list - Defect reporting | Collision, breakdown | 2 2 3 | 3 3 3 | 6 6 9 | - Up to date logbook system kept in vehicle showing information highlighted in CVT: 013 as a minimum – completed by the driver for every trip - Monthly safety check list (CVT: 014) completed by the Vehicle Trust / driver every month - Defect report form (CVT: 015) completed for every vehicle defect | 1 1 1 |  | 3 3 3 |  | 3 3 3 |
| 3 | - Emergency contacts - Vehicle emergency procedure card - Breakdown service | Unable to get help in an emergency, unable to get passengers home in a timely manner | 2 2 2 | 2 3 2 | 4 6 4 | - Emergency contacts: - where driver manages the system – emergency contact for each passenger kept in vehicle for each trip - where Vehicle Trust contact manages the system – emergency contact for each passenger kept to hand by Vehicle Trust contact; driver to ensure they carry Vehicle Trust contact name and number - Vehicle emergency procedure card (CVT: 021) displayed in each vehicle at all times Vehicle Trust are encouraged to carry a camera (mobile phone/dashcam) so any incidents can be recorded - The Vehicle Trust is a member of a breakdown service and the service has been made aware that disabled people that need transporting to different destinations, may be carried | 1 1 1 |  | 2 2 2 |  | 2 2 2 |
| 4 | Valid driving license | Collision, breakdown | 3 | 3 | 9 | All drivers have appropriate license that is checked …… by the Vehicle Trust | 1 |  | 3 |  | 3 |
| 5 | Disclosure check | Inappropriate behaviour | 2 | 2 | 4 | All drivers have completed a police disclosure check for the Vehicle Trust | 1 |  | 2 |  | 2 |

| Hazard |  | Accident | L | S | RF | Control Measures |  | L |  | S | RF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | - Induction training - Moving and handling - Refresher training | Collision due to misuse of equipment, breakdown, falls, injuries from supporting passengers | 3 2 2 2 2 | 3 3 3 3 3 | 9 6 6 6 6 | - All new drivers to receive an induction (see CVT: 016for details) from a competent person from their Vehicle Trust – this can be an experienced driver All new drivers must complete Advance driver…………training – certificate is transferable from other organisations All existing drivers however experienced who do not wish to complete Advance Driver Training must be willing to sign the Transport Induction Form (CVT016) stating that they can perform their duties in line with the standards set out by……………… - Drivers must complete a moving and handling course relevant to their work i.e. manoeuvring wheelchairs, lifting and carrying vehicle seats, assisting passengers Drivers who wish to physically lift passengers must complete appropriate training with a recognised body first Where moving and handling cannot be avoided; drivers must be trained to complete the tasks in line with manufacturers instructions i.e. handling wheelchairs or vehicle seats - Refresher training: - Induction: every …… – drivers - PSV training: every …….. – minibus drivers - Moving and handling training: every …….. – drivers | 1 1 1 1 1 |  | 3 3 3 3 3 |  | 3 3 3 3 3 |
| Hazard |  | Accident | L | S | RF | Control Measures |  | L |  | S | RF |
| 7 | - Fire fighting equipment - Inspection and maintenance - Fire prevention - Training - Personal protective equipment | Fire within the vehicle, passengers not knowing what action to take or unable to escape | 3 3 3 3 3 | 3 3 3 3 3 | 9 9 9 9 9 | - Clearly marked Water or Foam fire extinguisher in the vehicle; - Fire extinguishers checked daily to ensure not used or damaged. Competent person to inspect and tag the fire extinguishers annually - No smoking in branch vehicle. No spare fuel to be stored in branch vehicle. If oxygen cylinders transported, they are stored securely Driver know that they should never try to tackle an engine fire – just evacuate the vehicle - All drivers trained in the use of fire extinguishers All drivers trained in evacuation procedures including briefing passengers on what will happen if there is an emergency - Hi-vis vest to be stored in the vehicle and worn by the driver whenever they are operating the vehicle | 2 1 1 1 1 |  | 3 3 2 2 2 |  | 6 3 2 2 2 |
| 8 | First aid | Unable to treat injured personnel | 2 | 2 | 4 | A clearly marked first aid kit for treating minor injuries must be kept in the vehicle If a passenger is injured or becomes ill the driver will take control and will: - nominate individual to look after other passengers in the vehicle - nominate individual to call an ambulance and guide it to the patient and/ or vehicle - clear the area around the patient and keep them comfortable The driver must carry a fully charged mobile phone to summon the emergency services if needed | 1 |  | 2 |  | 2 |
| Hazard |  | Accident | L | S | RF | Control Measures | L |  | S |  | RF |
| 9 | Passenger lift/ hoist | Failure of passenger lift/ hoist, slips trips, falls from height, trap injuries | 3 | 3 | 9 | Passenger lift/ hoist must be inspected by a competent person every …months in line with Worksafe NZ https://worksafe.govt.nz/topic-and-industry/load-lifting-and-rigging/? Clear instructions for set up and operation of the lift/ hoist must be displayed inside the vehicle All drivers must be trained to set up and operate the lift/ hoist and must set up/ operate the lift/ hoist in line with this training and in conjunction with the manufacturer’s instructions at all times Driver check equipment is stored securely before moving off | 1 |  | 3 |  | 3 |
| 10 | - Ramp - Pulley system | Falls from height, slips, trips, wheelchairs tipping, moving and handling injuries | 2 3 | 3 3 | 6 9 | - Ramp must be of shallow gradient with raised edges and a non-slip surface. It must be maintained in line with the manufacturers instructions - Pulley system and any other associated equipment used for moving people must be inspected by a competent person every … months in line with Worksafe NZ https://worksafe.govt.nz/topic-and-industry/load-lifting-and-rigging/? All drivers must be trained to set up and operate the ramp and associated equipment and must set up/ operate the ramp and associated equipment in line with this training and in conjunction with the manufacturer’s instructions at all times Drivers check equipment is stored securely before moving off | 1 1 |  | 2 3 |  | 2 3 |
| 11 | Seatbelts and wheelchair restraints | Falls, wheelchairs tipping, trap injuries | 2 | 2 | 4 | Vehicle has suitable and sufficient wheelchair and passenger restraint equipment All passengers face either the front or the rear of the vehicle As part of the induction drivers to be trained to set up and operate the restraint equipment | 1 |  | 2 |  | 2 |
| 12 | Fuel cut off | Accelerated fire as fuel pumping into engine | 3 | 3 | 9 | If the vehicle has a fuel cut off Device ensure that, it is clearly marked and all drivers and escorts know how and when to activate it | 1 |  | 3 |  | 3 |
| Hazard |  | Accident | L | S | RF | Control Measures | L |  | S |  | RF |
| 13 | Welfare: - Substance abuse - Health issues that may affect driving - Tiredness - Lone working | Collision due to tiredness, distractions, substance abuse, driver suffering a medical emergency Inappropriate behaviour | 3 2 2 2 2 | 3 3 2 2 2 | 9 6 4 4 4 | - Drivers must never be under the influence of alcohol or drugs - Drivers must declare any health conditions that may affect driving to the branch; these must be reported to the insurance company for guidance - The Vehicle Trust committee must support volunteer drivers if they refuse to drive because they feel tired and unable to concentrate - Drivers to have access to a mobile phone in case of emergency, but must not use it while driving If the driver is threatened and no passengers being carried; the perpetrators must be allowed free access to what they want If the driver is alone in the vehicle for long periods, they must regularly communicate with a Vehicle Trust contact such as a branch contact, a friend or family member | 1 11 1 1 |  | 1 11 2 2 |  | 1 11 2 2 |
| Hazard |  | Accident | L | S | RF | Control Measures | L |  | S |  | RF |
| 14 | Driving regular routes - Environment - Weather - Planning the journey: | Wheelchair tipping, moving and handling injuries, slips, trips, getting lost or stuck in traffic, not able to carry new passengers or those with new equipment, passengers being injured outside the minibus | 2 2 2 3 | 3 3 3 3 | 6 6 6 9 | - Avoid steep slopes and rough ground when loading vehicles - If necessary to drive in severe weather, keep speed dependant on conditions - When planning journey: - check local radio for roadwork/ accident; have back up route - ensure all the necessary equipment is available and there is somewhere to store excess equipment and bags - if new passenger or existing one with a new wheelchair, the driver should check that they can accommodate this - if possible pick up and set down passengers in a safe place; watch until they are safely home - for emergency contacts see section 3a | 1 1 1 1 |  | 3 3 1 3 |  | 3 3 1 3 |
| Hazard |  | Accident | L | S | RF | Control Measures | L |  | S |  | RF |
| 15 | Driving regular routes - Handling wheelchairs - Passenger lift/ hoist/ ramp - Assisting passengers | Wheelchair tipping, moving and handling injuries, slips, trips, passengers being injured outside the minibus | 3 3 2 | 3 3 2 | 9 9 4 | - See section 6d - See section 9 and 10 - See section 6d and 13e | 1 1 1 |  | 3 3 2 |  | 3 3 2 |
| 16 | Day trips or holidays - Environment - Weather - Planning the journey: | Wheelchair tipping, moving and handling injuries, slips, trips, getting lost or stuck in traffic, not able to carry new passengers or those with new equipment, passengers being injured outside the minibus | 2 2 2 3 | 3 3 3 3 | 6 6 6 9 | - Avoid steep slopes and rough ground when loading vehicles - If necessary to drive in severe weather, keep speed dependant on conditions - When planning journey: - check for road works and adverse weather conditions - limit the amount of luggage passengers can bring. Ensure necessary equipment available and there is suitable secure storage for excess equipment and luggage without blocking emergency routes and exits - have a passenger list on the bus highlighting personal contact details and any special needs. Ensure a branch contact has this information in case of an emergency - if new passengers or an existing one with new wheelchair the driver should check that they can accommodate this - if possible pick up and set down passengers in a safe place; watch until they are safely home - ensure facilities available so regular breaks can be taken. Let passengers know the distance between breaks - driver to have regular breaks (….minutes every two hours); consider whether extra drivers are needed if long distances being covered | 1 1 1 1 |  | 3 2 1 3 |  | 3 2 1 3 |
| Hazard |  | Accident | L | S | RF | Control Measures | L |  | S |  | RF |
| 17 | Vehicle Trust lending out its vehicle to members - Insurance - Log book system/ safety checks/ defect reporting - Emergency contacts/ Breakdown service - Valid driving license - Training - Welfare | Collision, breakdown, injuries to passengers, inappropriate behaviour, failure of equipment due to misuse, falls from height, slips and trips Driver taken ill or becoming incapacitated | 2 2 2 3 3 3 1 | 3 3 2 3 3 3 3 | 6 6 4 9 9 9 3 | - Drivers insured for all purposes for which the vehicle is to be used - Driver borrowing the vehicle to complete log book; branch contact to complete defect report form where needed Driver borrowing vehicle should follow systems as laid out in section 2 of this risk assessment - Driver borrowing vehicle; briefed on action to take in emergency by Vehicle Trust contact and given details of breakdown service - Vehicle Trust contact to check that driver has appropriate license - The Vehicle Trust contact must provide training to the driver borrowing the vehicle that covers: - General use of vehicle e.g. refuelling, fuel cut off device etc. - Use of any equipment including wheelchair and passenger restraint systems, lifts/ hoists, ramps, pulley systems etc. - Action to take in an emergency (see CVT 021 for details and section 13 of this risk assessment for first aid incident) - Vehicle Trust contact to brief driver on any welfare guidelines that are relevant (see section 13 of this risk assessment for details) | 1 1 1 1 1 1 1 |  | 3 3 2 3 3 3 1 |  | 3 3 2 3 3 3 1 |
| Hazard |  | Accident | L | S | RF | Control Measures | L |  | S |  | RF |
| 18 | Refuelling Incorrect fuel Spilling fuel | Financial impact fire, slips, trips and falls | 2 3 | 2 3 | 4 9 | Check correct pump is used and that the nozzle fits the vehicle Driver to operate the pump in line with the fuel station instructions No smoking in the area when carrying out refuelling Mobile phones should be left in the vehicle and ideally switched off If fitted; individual filling up knows where fuel cut off device is located Under no circumstances try to fight a fuel or engine fire | 1 1 |  | 2 3 |  | 2 3 |